LED Moving head wash

GHOST 760 RGBW B-EYE

Manual

Content

1. Main technical parameters	2

2.Lighting spectrum	3

3.safety warnings	3

4.Power and control signal connections	4

5.operating instructions…………………………………………………………………………. 5

5.1display functions	5

5.2equipment control	5

5.3system settings	5

5.4equipment address	7

5.5 system information	7

6.DMX512 channel definition	8

6.1 【channel table】	8

7.configuration instructions	15

- Main Technical Specifications

Input voltage:100～240VAC 50/60Hz

Rated power:580W

Max power：620W

Light source: 7pcs *60W RGBW (4 in 1)COB LED

Lamp life: greater than 100000 hours

Beam angle: 2.8°～ 55°

Illuminance: 13575Lux @8m 2.8°

Scan: horizontal scan 540°, vertical scan 220°

Scan accuracy: 16bit precise scanning

Display: 2.4-inch LCD color display

Control modes:DMX512, master-slave, manual

Standard control: 13/24/52 channels optional DMXinput/output interface:3PIN/5PIN pin/socket connector

Noise: 42dB

Protection rating:IP20

Operating ambient temperature:-20～40℃

Net weight: 18Kg

Dimensions:406*305*492mm

- Lighting spectrum

3.Safety Warnings

Important Reminder：

●Installation and maintenance of this equipment must be carried out by personnel qualified in electronics.

●When installing by suspension, a safety cable must be installed.

●This equipment has undergone strict inspection before leaving the factory.

●Users must follow the safety rules and precautions in this manual below. ●The manufacturer will not be liable for any accidents or losses caused by improper operation not in accordance with this manual or by unauthorized modifications to the equipment.

Key points:

●The supply voltage must match the indicated voltage; ensure the equipment is used within the specified voltage range.

●The power input connector of this luminaire must not be connected to other electrical devices.

●This fixture'sDMX512signal line must not be connected to other electrical equipmentDMX512signals.

●Before use, please ensure the power switch outlet grounding conductor is in good condition.

●The yellow-green wire is the grounding conductorTo ensure safety and proper operation of the equipment, please choose a quality-certified power outlet or cut off the plug and connect directly to the switch.

●During installation, normal use, and movement,please avoid hard impacts to this equipment.

4.Power and Control Signal Connections

□This product uses an input power cable of2.5mm² copper core wire. □When using this product’sDMX512signal input/output connector cables, do not exceed20devices. For signal cables between theDMXconsole and devices or between devices, if the length exceeds50meters, a signal amplifier of one stage must be added.

□It is recommended to connect a terminator plug to the signal output socket of the last device at the end of each signal transmission line. The terminator plug is a plug with a 120Ω/1W resistor soldered between pins 2 and 3 of the male connector. Its function is to reduce reflection interference caused by an open circuit at the end of the control signal cable.

- Operation Method

5.1 Display Functions

Button Functions
1.[ENTER] Enter the next level menu, allow modification or setting of options;
2.[UP] Move cursor up;
3.[DOWN] Move cursor down;
4. [MENU] Return to previous menu;
5. Enter standby state if no key is pressed within 50 seconds; Turn off backlight if no key is pressed within 10 seconds after standby; Press [UP] key or [DOWN] key to turn on backlight; Press [ENTER] key or [MENU] key to return to the screen before standby.

5.2 Equipment Control

- Confirm【System Settings】/【Operating Mode】has been changed to [MAN]Manual Mode;

- In the main menu, move the cursor to the “Device Control”icon and press[ENTER]to enter the menu；

- Press[UP]or[DOWN]to cycle through the data channels you want to modify;

- Press[ENTER]to enable data modification;

- Press [UP] or [DOWN] to cycle through and change the data;

- Press[MENU] to prevent data modifications;

- Repeat steps 3-6 modify other data channels;

- See the “Control Channel Table”for the data channel layout.

5.3 System Settings

- Move the cursor to [System Settings] in the main menu and press [ENTER] to enter the menu;

- Press [UP] or [DOWN] to select the item you want to modify;

- Press [ENTER] to cycle through settings;

- Press the [MENU] key to exit;

5. For detailed settings see the table below.

| Options | Value |
| --- | --- |
| Operating mode | DMX、M/S、MAN |
| Channel Settings | Reduced、Standard、Extended |
| Host Program | None、Prog1、Prog2、Prog3 |
| Language Selection | Chinese、English |
| Screen Rotation | Normal、Inverted |
| Dimming Curve | Linear、Square、I-Square、S-Shape |
| Horizontal flip | Normal、Inverted |
| Vertical flip | Normal、Inverted |
| Motion swap | Normal、Swapped |

DMX Mode

- Confirm [System Settings]à[Operation Mode] has been changed to [DMX] communication mode;

- Confirm the moving head controller is correctly connected to thisDMXdevice;

- Confirm the device address is set correctly;

- When using the fixture library, ensure the channel mode matches the fixture library.

M/S Mode

- Please disconnect the device from the moving head controller; if connected, it may not operate properly; 2.Determine the unique master device on the communication link;

- Confirm the master device in 【System Settings】à【Operation Mode】 has been changed to [M/S] Master mode; 4.Select the master device in 【System Settings】à【Master Program】;

- [M/S] Master mode is supported only by products in this series.

- In [M/S] mode, the number of devices connected to this product does not exceed 12.

MAN Mode

- Confirm [System Settings]à[Run Mode] has been changed to [MAN] Manual mode;

- Move the cursor to [Device Control] in the main menu and press [ENTER] to enter the menu;

- Press[UP] or[DOWN] to cycle through the data channels to be modified;

- Press[ENTER] to allow modification of the data; 5.Press[UP] or[DOWN] to cycle through and modify the data;

6.Press[MENU] to disallow modification of the data; 7.Repeat steps 3–6 to modify other data channels;

5.4Equipment Address

- Move the cursor to the "Device Address" icon in the main menu and press [ENTER] to enter the menu;

- Press [UP] or [DOWN] to cycle through and change the address;

3. Press [MENU] to save and exit.

5.5System Information

- In the main menu, move the cursor to the "System Information" icon and press [ENTER] to enter the menu; 2. Displays device address, channel mode, LED panel temperature, cumulative allowed time, and more;

3. Press [MENU] to exit.

6  DMX512 Channel Definitions

This fixture occupies multiple DMX512 signal channels. Specific channel functions are as follows:

6.1 【Channel Table】

| M-L0760YZ - 7pcs 60W Bee Eye Channel PanelV1.0.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DMX channel´s functions and their values (52 DMX channels): |  |  |  |  |  |
| Mode/Channel |  |  | Value | Function |  |
| Reduced | Standard | Extended |  |  |  |
| 1 | 1 | 1 |  | Dimmer intensity : |  |
|  |  |  | 0-255 | Intensity 0 to 100% | Dimmer from 0 to 100% |
| 2 | 2 | 2 |  | Red LED : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | Red from 0～100％ |
| 3 | 3 | 3 |  | Green LED : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | Green from 0～100％ |
| 4 | 4 | 4 |  | Blue LED : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | Blue from 0～100％ |
| 5 | 5 | 5 |  | White LED : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | White from 0～100％ |
| 6 | 6 | 6 |  | Strobe : |  |
|  |  |  | 0-255 | 1-25Hz | Strobe from slow to fast |
| 7 | 7 | 7 |  | Pan Movement 8bit : |  |
|  |  |  | 0-255 | Pan Movement | Horizontal Scan |
| 8 | 8 | 8 |  | Pan Fine 16bit |  |
|  |  |  | 0-255 | Fine control of Pan movement | Horizontal Scan Fine Adjustment |
| 9 | 9 | 9 |  | Tilt Movement 8bit : |  |
|  |  |  | 0-255 | Tilt Movement | Vertical Scan |
| 10 | 10 | 10 |  | Tilt Fine 16bit |  |
|  |  |  | 0-255 | Fine control of Tilt movement | Vertical scan micromovement |
| 11 | 11 | 11 |  | Speed Pan/Tilt movement: |  |
|  |  |  | 0-255 | max to min speed | Scanning speed from fast to slow |
| 12 | 12 | 12 |  | Focus : |  |
|  |  |  | 0-255 | Focus adjustment | Magnification from large to small |
| 13 | 13 | 13 |  | Rotation : |  |
|  |  |  | 0-127 | 0-60° position | 0-60° positioning |
|  |  |  | 128-191 | Zoom Rotation Forward min to max speed | Rotate counterclockwise from slow to fast |
|  |  |  | 192-255 | Zoom Rotation Reverse min to max speed | Rotate clockwise from slow to fast |
| - | 14 | 14 |  | Hue : |  |
|  |  |  | 0-255 | 0-360° | Hue |
| - | 15 | 15 |  | Hue Fine : |  |
|  |  |  | 0-255 | 0-10° | Tone microadjustment |
| - | 16 | 16 |  | Saturation : |  |
|  |  |  | 0-255 |  | Dark to light |
| - | 17 | 17 |  | CCT : |  |
|  |  |  | 0-255 | 2800-10000K | Color Temperature |
| - | 18 | 18 |  | Color Shade : |  |
|  |  |  | 0-255 | 1-240s |  |
| - | 19 | 19 |  | Color Preset : |  |
|  |  |  | 0-255 |  |  |
| - | 20 | 20 |  | Chase Patterns : |  |
|  |  |  | 0--5 | Led trun off | Close |
|  |  |  | 6--10 | Chase 1 | Fixed Scene1 |
|  |  |  | 11--20 | Chase 2 | Fixed Scene2 |
|  |  |  | 20--30 | Chase 3 | Fixed Scene3 |
|  |  |  | 31--40 | Chase 4 | Fixed Scene4 |
|  |  |  | 41--50 | Chase 5 | Fixed Scene5 |
|  |  |  | 51--60 | Chase 6 | Fixed Scene6 |
|  |  |  | 61--70 | Chase 7 | Fixed Scene7 |
|  |  |  | 71--80 | Chase 8 | Fixed Scene8 |
|  |  |  | 81--90 | Chase 9 | Fixed Scene9 |
|  |  |  | 91--100 | Chase 10 | Fixed Scene10 |
|  |  |  | 101--110 | Chase 11 | Fixed Scene11 |
|  |  |  | 111--120 | Chase 12 | Fixed Scene12 |
|  |  |  | 121--130 | Chase 13 | Fixed Scene13 |
|  |  |  | 131--140 | Chase 14 | Fixed Scene14 |
|  |  |  | 141--150 | Chase 15 | Fixed Scene15 |
|  |  |  | 151--160 | Chase 16 | Fixed Scene16 |
|  |  |  | 161--170 | Chase 17 | Change Scene1 |
|  |  |  | 171--180 | Chase 18 | Change Scene2 |
|  |  |  | 181--190 | Chase 19 | Change Scene3 |
|  |  |  | 191--200 | Chase 20 | Change Scene4 |
|  |  |  | 201--210 | Chase 21 | Change Scene5 |
|  |  |  | 211--220 | Chase 22 | Change Scene6 |
|  |  |  | 221--230 | Chase 23 | Change Scene7 |
|  |  |  | 231--240 | Chase 24 | Change Scene8 |
|  |  |  | 241--250 | Chase 25 | Change Scene9 |
| - | 21 | 21 |  | Chase Speed : |  |
|  |  |  | 0-255 | Slow toFast | Forward scene speed from slow to fast |
| - | 22 | 22 |  | Chase Fade : |  |
|  |  |  | 0-255 | Fade Chase | Gradient Speed |
| - | 23 | 23 |  | System Function : |  |
|  |  |  | 000-019 | None Function |  |
|  |  |  | 020-039 | Linear Dimmer Curve |  |
|  |  |  | 040-059 | Square Dimmer Curve |  |
|  |  |  | 060-079 | I-Square Dimmer Curve |  |
|  |  |  | 080-099 | S-Shape Dimmer Curve |  |
|  |  |  | 100-255 | None Function |  |
| - | 24 | 24 |  | System Control : |  |
|  |  |  | 000-249 | None Function |  |
|  |  |  | 250-255 | Reset the fixture |  |
| - | - | 25 |  | Red LED -array 1 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED1red zone from0～100％ |
| - | - | 26 |  | Green LED -array 1 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED1green zone from 0～100％ |
| - | - | 27 |  | Blue LED -array 1 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED1blue zone from 0～100％ |
| - | - | 28 |  | White LED -array 1 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED1 white zone from 0～100％ |
| - | - | 29 |  | Red LED -array 2 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED2 red zone from 0～100％ |
| - | - | 30 |  | Green LED -array 2 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED2green zone from 0～100％ |
| - | - | 31 |  | Blue LED -array 2 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED2bule zone from 0～100％ |
| - | - | 32 |  | White LED -array 2 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED2white zone from 0～100％ |
| - | - | 33 |  | Red LED -array 3 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED3 red zone from 0～100％ |
| - | - | 34 |  | Green LED -array 3 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED3 green zone from 0～100％ |
| - | - | 35 |  | Blue LED -array 3 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED3 blue zone from 0～100％ |
| - | - | 36 |  | White LED -array 3 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED3 white zone from 0～100％ |
| - | - | 37 |  | Red LED -array 4 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED4 red zone from 0～100％ |
| - | - | 38 |  | Green LED -array 4 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED4 green zone from 0～100％ |
| - | - | 39 |  | Blue LED -array 4 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED4 blue zone from 0～100％ |
| - | - | 40 |  | White LED -array 4 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED4 white zone from 0～100％ |
| - | - | 41 |  | Red LED -array 5 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED5 red zone from 0～100％ |
| - | - | 42 |  | Green LED -array 5 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED5 green zone from 0～100％ |
| - | - | 43 |  | Blue LED -array 5 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED5 blue zone from 0～100％ |
| - | - | 44 |  | White LED -array 5 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED5 white zone from 0～100％ |
| - | - | 45 |  | Red LED -array 6 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED6 red zone from 0～100％ |
| - | - | 46 |  | Green LED -array 6 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED6 green zone from 0～100％ |
| - | - | 47 |  | Blue LED -array 6 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED6 blue zone from 0～100％ |
| - | - | 48 |  | White LED -array 6 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED6 white zone from 0～100％ |
| - | - | 49 |  | Red LED -array 7 : |  |
|  |  |  | 0-255 | Red ( 0-Black , 255-100% Red ) | LED7 red zone from 0～100％ |
| - | - | 50 |  | Green LED -array 7 : |  |
|  |  |  | 0-255 | Green ( 0-Black , 255-100% Green ) | LED7 green zone from 0～100％ |
| - | - | 51 |  | Blue LED -array 7 : |  |
|  |  |  | 0-255 | Blue ( 0-Black , 255-100% Blue ) | LED7 blue zone from 0～100％ |
| - | - | 52 |  | White LED -array 7 : |  |
|  |  |  | 0-255 | White ( 0-Black , 255-100% White ) | LED7 white zone from 0～100％ |

7. Configuration Instructions

Power connection cable1piece

Signal connection cable1piece

Safety rope1piece

Instruction manual1copy

Warranty card1sheet

Lamp body mounting plate 2 pieces